Galvashield XP4

High performance, tied-on sacrificial anode units, to provide cathodic protection to corrosion formation in adjacent areas to patch repairs

*1.0

Sacrificial Anodes

1.10

Anode placement

The sacrificial anode units shall be positioned at centres determined by the Engineer, around / along the repair boundary, dependant on the reinforcement steel configuration and repair geometry. Confirm actual layout with the Engineer prior to fixing any units in place.

1.11

To those locations and spacings, determined by the Engineer, the sacrificial anode units shall be attached immediately following preparation and cleaning of the steel reinforcement.

The location of the sacrificial units shall be as close as practical to the edge of the broken out repair zone, in specially excavated rebate(s) if required.

1.20

Anode Units

1.21

The sacrificial anode(s) shall be a zinc based block surrounded by a specially formulated cementitious mortar. This mortar shall have a pore solution pH which is sufficiently high for corrosion of the anode to occur and for passive film formation on the anode to be avoided, as described in patent no. PCT/GB94/01224.

1.22

Galvashield XP4 from Parchem Construction Supplies meets the product criteria and is an approved product.